High Power LED PAR 54x3W RGBW

A-6/ Cast  Al

1.Picture Part

LED

A

B

C

D

A.MENU

B. UP

E.LED WINDOW

C. DOWN

D. ENTER

Operate Manual :Press A can get 8 kind of different function effect show , The first two NO. are instead of the Function show ,(Follow the Function) The after of two NO. are instead of the Statue of the  Address ID, Or speed , Parameter .Press B or C can modify the Parameter , Press D Enter.

2.Led Window Display Checking Address.(All of the function are need to Press D for make enter and Confirm . )

| No. | Display | Function Introduce |
| --- | --- | --- |
| 1. | D001 | 7ch ID (001-512)B. C Increase & Decrease ID |
| 2. | CC50 | 7Color Jump （01-99） B .C Change Jump Speed |
| 3. | CP50 | 7Colors, changes of pulse, Gradual changes,)01-99) B.C Modify the change speed |
| 4. | CS50 | 7 Colors Choice (01-99) B .C Modify Color |
| 5. | DE50 | 7 Color Combination Change (01-99) B.C Modify Combination Change Speed |
| 6. | BE50 | 7 Colors Sound Change(01-99) B.C Modify the sound Change Speed |

3.DMX 512 CH

| NO. | Function | Introduce |
| --- | --- | --- |
| CH 1 | General Adjust light | R.G.B. General Adjust . Liner light , From dark to Bright |
| CH 2 | General Strobe | R.G.B General Strobe . From slow to fast |
| CH 3 | Function Choice | 0-50 DMX 7CH Control, 51-100 Jump change , 101-150 : Gradual changes, changes of pulse , 151-200:Jump. Gradual changes, changes of pulse, 201-255 , Sound Control |
| CH 4 | Function speed | Function Speed , From Slow to Fast |
| CH 5 | R Adjust | R Adjust, Liner Light, From dark to Bright |
| CH 6 | G Adjust | G Adjust , Liner Light, From dark to Bright |
| CH 7 | B Adjust | B Adjust, Liner Light , From dark to Bright |
| CH8 | W Adjust | W Adjust,Liner Light,From dark to Bright |

.4.Function Special: Simple operation, humanized design, adjustable light flicker free vibration, suitable for photography, photography, television and other light demanding situations