200W Beam Moving Head

Please read the manual carefully before using

Contents

1 Operation Mode

2 Interface Introductions

3 Channel Modes

4 Function Reference

5 Securities

6 Caution

- Operation Introduction

| Touching | Runner | Function |
| --- | --- | --- |
| Up Button | turn right | select/edit |
| Down Button | turn left | select/edit |
| Enter Button (OK Button) | press the runner | function running edit opening edit escape |
| Back Space key | press the runner on the back menu | return to the ad menu |

To take a "modify the DMX address number" example for explain the function of touching:

1Touch Settings button in the main interface, enter the setup interface

2 Right side of the four touch buttons are "on", "down", "return"

3 Press the Up or Down keys, select dmx address code

4 Press the OK button to edit

5 Press the OK button to modify the DMX address (immediate save and apply the new

address automatively

6 Press the Enter key to esc edit state (or this step can be omitted)

7 Press the Backspace key to return to the main menu

If using a wheel, then the process is as follows

1 In the main interface, turn the runner, select Set Up button

2 Press the wheel and then release into the setting interface

3 In the Setting interface, turn the wheel, select DMX Address

4 Press the wheel and then release into the setting interface

5 Turn the wheel to esc edit state

6 After modify the address code, press the wheel then release to esc edit state

7Turn the wheel to select return, press the button and then release back to the main interface

2. Interface Setting

1) Master Interface

Top right corner of the three buttons are for language switching and screen rotating.

The bottom of four buttons corresponding sub-interface should see below.

The functions of sub-interface are mainly about touch key, and if you choose runner, the corresponding relationship are as follows:

Press "up" key: turn left

Press "down" key: turn right

Press "enter" key: turn down the wheel

Dialogue box of the runner operation

Press “enters" to turn down the wheel. Press "cancel" if not.

2) Interface setting

| Option | Information |  |
| --- | --- | --- |
| Operation Mode | DMX | Slave state: receive the DMX signal |
|  | Automation | Master state: operate automatively, sending DMX signal to slave Caution: You can check light effect only when the light on and enter an automatic state |
| DMX Address | 1~512 | Press “Enter” to edit. select hundred, press “up” ”down” to modified the address. Press “Enter “to select decade to edit. One more time to press |
| Channel Mode | 15 |  |
|  | 18 |  |
| X Reverse | ON/OFF |  |
| Y Reverse | ON/OFF |  |
| XY Reverse | ON/OFF |  |
| XY Exchange | OFF |  |
|  | ON | To exchange XY channel(including detail correct) |
| XY Encoder | ON | Check the missing step by optocoupler and correct. |
|  | OFF | Correct the position without using the optocoupler. |
| Non-signal DMX | Save | Operate under the original state |
|  | Clear | Reset motor, stop working |
| Screen Protection | ON | Backlight off after 30s vacant |
|  | OFF | Keep lighting under backlight |
| Lighting Boot | ON | Light on as soon as the power on, then reverse |
|  | OFF | Reverse as soon as the power on(use the menu or console to light up) |
| Default Setting Recovery |  | Press “Enter” to check the dialogue, then press “Enter” to recover default setting |

- Information Interface

| Option | Information |
| --- | --- |
| Software | The newly version |
| Total Time | Total using time(accurate to minute) |
| Single Time | Total time for single using(accurate to minute) |
| DMX Channel Value | Check the channel value and percentage under this sub-interface |
| System error Record | If the red light ERR sparks, you should check the detail information by enter sub-interface cause the device must went something wrong |

4) Function Reference

This is only for controlling the device, not belong to the slave state(do not receive DMX signal),neither the master state(do not send DMX signal)

| Option | Information |  |
| --- | --- | --- |
| Reset | Press “Enter” to check the dialogue, press “Enter” again to enter reverse interface, all the motor will reverse |  |
| Color Wheel | 0~255 | Press “Enter” to edit. Press “up””down” to change channel value when select hundred position. Press “Enter” again to select decade to edit.The same way to edit individual value. One more time press “Enter” to exit |
| …… | 0~255 |  |
| Gobo wheel Speed | 0~255 |  |
| Lamp Control | ON |  |
|  | OFF |  |

5)Advanced Interface

The code “up””down””up””down” respond to wheel “left””right””left””right”

Touching Process: First time press “up” button (the first”*” display), the second time press “up” button(the second “*” display), the third time to show “*” is the same. Finally to press “Enter “to verify the password

Rotate Wheel Process: Right turn forward (the first “*” display), left turn backward (the second “*” display), the third time to show “*” in the same way to step 1, the forth time to show “*”in the same way to step 2.Finally press the wheel to verify the password.

| Option | Information |
| --- | --- |
| Touching screen test | Enter testing interface, touch the place according to the cross mark until receive OK respond 4 times.In case of testing failure and the endless process, press “Enter” to exit. |
| Reverse Test | When enter sub-interface, adjust the XY motor reverse position to lessen the hardware mistakes. You have to edit one by one for single step cause it is different from address and channel value. Caution: you are not suggest to test when the motor is working. If it is necessary, please reverse the device before testing. |

- Channel Modes

| 1 | Color Wheel | Color Wheel |
| --- | --- | --- |
| 2 | Light Cut/Strobe | Light Cut/Strobe |
| 3 | Dimmer | Dimmer |
| 4 | Gobo Wheel | Gobo Wheel |
| 5 | Prism | Prism |
| 6 | Prism Rotate | Prism Rotate |
| 7 |  |  |
| 8 | Atomization mirror | Atomization mirror |
| 9 | Focus | Focus |
| 10 | X axis | X axis |
| 11 | X Fine tuning | X Fine tuning |
| 12 | Y axis | Y axis |
| 13 | Y Fine tuning | Y Fine tuning |
| 14 | XY Speed | XY Speed |
| 15 | Reverse | Reverse |
| 16 | Lamp Control | Lamp Control |
| 17 |  | Gobo Wheel/Speed |
| 18 |  | Dimmer-Prism-Atomization Speed |

4. Function Reference

Channel 1

| DMX Value | Effect | Remark |
| --- | --- | --- |
| 255 | Brightness100 |  |
| …… | …… |  |
| 0 | Brightness0 |  |

Channel 2

| DMX Value | Effect | Remark |
| --- | --- | --- |
| 252-255 | Light Shut | Combination Channel 1 |
| 239-251 | Radom Fast Strobe |  |
| 266-238 | Radom Normal Strobe |  |
| 213-225 | Radom Slow Strobe |  |
| 208-212 | Shutter On | Combination Channel 1 |
| 207 | Pulse Fast Strobe |  |
| …… | …… |  |
| 108 | Pulse Slow Strobe |  |
| 104-107 | Shutter On | Combination Channel 1 |
| 103 | Fast Strobe |  |
| …… | …… |  |
| 4 | Slow Strobe |  |
| 0-3 | Shutter Off |  |

Channel 3

| DMX Value | Effect | Remark |
| --- | --- | --- |
| 255 | Fast Rotating | All of the color value is designed to be the multiple of 5, so that easy to remember. The percentage can be adjust, example: When display shows 5 that means white 50%, dark red 50%. When it shows 4, that means white 40%, dark red 60%. |
| …… | …… |  |
| 150 | Slow Rotating |  |
| 145 | Brown+White |  |
| …… | …… |  |
| 5 | White+Red |  |
| 0 | White |  |

Channel 4

| DMX Signal Value | Effect | Remark |
| --- | --- | --- |
| 255 | Gobo17 Shack Fast | Every 5 gobo response to one gobo |
| …… | …… |  |
| 251 | Gobo17 Shack Slow |  |
| 175 | Gobo1 Shack Fast |  |
| …… | …… |  |
| 171 | Gobo1 Shack Slow |  |
| 170 | Fast Rotating(Forward) |  |
| …… | …… |  |
| 135 | Slow Rotating(Reverse) |  |
| 130-134 | Stop(White) |  |
| 129 | Slow Rotating(Reverse) |  |
| …… | …… |  |
| 90 | Fast Rotating(Reverse) |  |
| 85 | Gobo17 | The value would be Multiples of 5 |
| …… | …… |  |
| 5 | Gobo1 |  |
| 0 | White |  |

Channel 5

| DMX Value | Effect | Remark |
| --- | --- | --- |
| 128-255 | Insert Prism |  |
| 0-127 | Remove Prism |  |

Channel 6

| DMX Value | Effect | Remark |
| --- | --- | --- |
| 255 | Rotating Fast(Forward) |  |
| …… | …… |  |
| 193 | Rotating Slow(Forward) |  |
| 191-192 | Stop |  |
| 190 | Rotating Slow(Reverse) |  |
| …… | …… |  |
| 128 | Rotating Fast(Reverse) |  |
| 0-127 | Prism Angle Adjust |  |

Channel 8

| DMX Value | Effect | Remark |
| --- | --- | --- |
| 128-255 | Atomization |  |
| 0-127 | Non-atomization |  |

Channel 9

| DMX Value | Effect | Remark |
| --- | --- | --- |
| 255 | Dimmer100 |  |
| ……. | …… |  |
| 0 | Dimmer0 |  |

Channel 10

| DMX Value | Effect | Remark |
| --- | --- | --- |
| 255 | Pan Movement 540 |  |
| …… | …… |  |
| 0 | Pan Movement 0 |  |

Channel 11

| DMX Value | Effect | Remark |
| --- | --- | --- |
| 255 | Pan set accuracy 100 |  |
| …… | …… |  |
| 0 | Pan set accuracy 0 |  |

Channel 12

| DMX Value | Effect | Remark |
| --- | --- | --- |
| 255 | Tilt Movement270 |  |
| …… | …… |  |
| 0 | Tilt Movement0 |  |

Channel 13

| DMX Value | Effect | Remark |
| --- | --- | --- |
| 255 | Tilt set accuracy 100% |  |
| ...... | …… |  |
| 0 | Tilt set accuracy 0% |  |

Channel 14

| DMX Value | Effect | Remark |
| --- | --- | --- |
| 255 | Pan/Tilt run in fast speed |  |
| …… | …… |  |
| 0 | Pan/Tilt run in fast speed |  |

Channel 15

| DMX Value | Effect | Remark |
| --- | --- | --- |
| 128-255 | Reset All the Motor | Reset the device after staying at the state for 5 seconds |
| 77-127 | Reset Main Motor(XY) |  |
| 26-76 | Reset Individual Motor |  |
| 0-25 | Non-Valid Area |  |

Channel 16

| DMX Value | Effect | Remark |
| --- | --- | --- |
| 101-255 | Turn On the Lamp | Operate the device after staying at the state for 5 seconds |
| 10-100 | Turn Off the Lamp |  |

5 Security

1. Please never try to project the beam onto substances for a long time to avoid getting fire. For this device is high light temperature product.

2. In order to the consumer’s profit and ensure the long life time of the device, we suggest that keep it dry in working during -20~30 temperature.

3. This is for situation the power is on and when the device is not indeed. Do not operate the device under dark mode for more than 20 minutes. You should first use the 14 channel under the third channel mode to turn off the light.(Please remind the lamp will be lighted up after 10 minutes when it was just turn off.)

4. The device needs about 20 minutes to be cooled after the power off.

5. Operate the device only after having familiarized with its functions. Do not permit operation by persons not qualified for operating the device. Most damages are the result of unprofessional operation.

6. Use the professional fitting when the device needs to be hanging up or change the details. Please contact relate factory if you can’t get the fitting.

6 Caution

After connect to Power Supply,Operate up to 255 on Channel 16 with DMX Controller,Then the lamp will be ON gradually. (Operate the device after staying at the state for 5 seconds.)

You can also set up "Burn on startup" on the touch screen on the light device.