LED Blinder light

User manual

- 1. Graphic part：

E

A．Function key

B．UP

C．DOWN

D．ENTER

E．LED DISPLAY

A        B         C        D

Instructions:

Press the function key A will cycle out of eight different functional effects, digital tube in which the first two digits represent the current function (see menu). After two digits to represent the kinds of functions in which the Address Code. Or speed parameters. Press B or C key to change its value. Press the D key to confirm.

- 2.  LED display window function table,

- after all of the features selected according to press the D key to confirm

| No. | display | function |
| --- | --- | --- |
| 1 | A001 | 8 channels address code，（001—512） B、C key:add and reduce the address code |
| 2 | H001 | 20 channels address code，（001—512） B、C key:add and reduce the address code |
| 3 | CC00 | 7 colors Transition，（01—99） B、C key:change the speed of Transition |
| 4 | CP00 | 7 colors Pulse change、Gradient，（01—99） B、C key:change the speed of Pulse change、Gradient |
| 5 | DE00 | 7 color combination of changes， （01—99） B、C key:change the speed of the combination |
| 6 | FF00 | Running of changes， （01—99） B、C key:change the speed of the Running |
| 7 | BEB1 | Sound active,color running changing mode1 |
| 8 | BEB2 | Sound active,color running changing mode2 |
| 9 | ID00 | DMX512 ID， （00—51）,ID=0: ID off After press”D” “B”、”C”can revise the DMX512 ID。ID=1--51: ID work ID will show 5 seconds when start light |
| 10 | FROM | SLAVE MODE |

- 3．8  DMX512 channels

| No. | function |  |
| --- | --- | --- |
| 1CH | Total dimmer | R、G、Btotal dimmer，Linear dimming, from dark to light |
|  | Total strobe | R,G,B total strobe,from slow to quick |
|  | Function select | 0--20：DMX 7CH control；21--70：Hopping change；71--120：Gradual change；121--170：Variable pulse；171--220：LED Running；221--240：sound active 1；241--255：sound active2 |
|  | Function speed | From slow to quick |
| 2CH | Red dimmer | Red dimmer,Linear dimming, from dark to light |
| 3CH | Green dimmer | Green dimmer,Linear dimming, from dark to light |
| 4CH | Blue dimmer | Blue dimmer,Linear dimming, from dark to light |
| 5CH | DMX ID | ID 1 5--9 ID 2 10--14 ID 3 15--19 ID 4 20--24 ........ ......... ID 51 255 THIS FUNCTION CAN CONTROL 50PCS LIGHTS MOST WHEN IN SAME DMX ID |

- 20 DMX512 channels

| No. | function |  |
| --- | --- | --- |
| 1CH | Total dimmer | R、G、Btotal dimmer，Linear dimming, from dark to light |
| 2CH | Total strobe | R,G,B total strobe,from slow to quick |
| 3CH | Function select | 0--20：DMX 7CH control；21--70：Hopping change；71--120：Gradual change；121--170：Variable pulse；171--220：LED Running；221--240：sound active 1；241--255：sound active2 |
| 4CH | Function speed | From slow to quick |
| 5CH | Red 1 dimmer | Red1 dimmer,Linear dimming, from dark to light |
| 6CH | Green1 dimmer | Green1 dimmer,Linear dimming, from dark to light |
| 7CH | Blue1 dimmer | Blue1 dimmer,Linear dimming, from dark to light |
| 8CH | Red 2 dimmer | Red2 dimmer,Linear dimming, from dark to light |
| 9CH | Green2 dimmer | Green2 dimmer,Linear dimming, from dark to light |
| 10CH | Blue2 dimmer | Blue2 dimmer,Linear dimming, from dark to light |
| 11CH | Red 3 dimmer | Red3 dimmer,Linear dimming, from dark to light |
| 。。。。。。。。 | 。。。。。。。。 | 。。。。。。。。。。。。。。。。。。。。。。。。。。。。。。。。。 |
| 20CH | DMX ID | ID 1 5--9 ID 2 10--14 ID 3 15--19 ID 4 20--24 ........ ......... ID 51 255 THIS FUNCTION CAN CONTROL 50PCS LIGHTS MOST WHEN IN SAME DMX ID |

- 5. Features:
   Operation is very simple, user-friendly design, dimming without flicker, jitter, etc. Can be adapted to photography, photography, television and other demanding applications on the lights.